# CADF2 – Software Setup Procedure (Configuration Guidelines)

## 1. Purpose

This section defines the software configuration guidelines required to ensure stable operation of the CADF2 system.

Incorrect configuration of backend files (especially shell scripts) may lead to issues such as terminal freezing, application crashes, or unstable behavior during long-duration operation. These problems are typically caused by incorrect IP addresses, ports, or flags. Therefore, all configurations must be handled carefully and should not be modified without proper understanding.

## 2. PC Application Configuration (CADF2_RUN.sh)

Before running the PC application, ensure that the latest released version of the executable is being used and that the same is correctly referenced in the CADF2_RUN.sh script.

In this script, the first IP address and port correspond to the hardware unit. This must be set correctly. Example format:

- Hardware IP: 192.168.1.100

- Port: 5001

If the hardware IP is unknown, confirm with the developer before making any changes.

The next IP and port correspond to the laptop running the application:

- Laptop IP: 192.168.1.101

- Port: 6001

This is followed by the GUI IP and port:

- GUI IP: 192.168.1.101

- Port: 7001

The GUI port must match the WebSocket port defined in the GUI configuration .json file.

There are additional IPs and ports present in the script which are reserved and not actively used. These must not be modified. If any changes were made previously, revert them back to their original values (as per reference image or initial setup).

Further in the script, configuration for the communication dongle (Amazon Basics 4G/5G module) is present:

- Master Dongle IP: 10.0.0.1

- Port: 8001

- Local Laptop Dongle IP: 10.0.0.2

- Port: 8002

For the Master Unit, both dongle IPs will be the same. Ensure correct configuration to maintain proper communication across units.

## 3. Remote Server Configuration (cadf2_server.sh)

Ensure that the latest version of the remote server application is used and correctly referenced in the cadf2_server.sh script.

In this script, configure the dongle IP and port of the Master Unit, for example:

- Master Dongle IP: 10.0.0.1

- Port: 8001

The remote server operates only on the master system and is responsible for communication with all other units.

A flag is also present in the script to control logging behavior:

- 1 → Enable debug prints

- 0 → Disable prints

Use logging (1) during debugging and disable (0) during normal operation.

## 4. GUI Configuration

Ensure the GUI application is updated to the latest version (Main_v1_0_1) and is correctly executed from its designated path.

Within the GUI folder, navigate to:

CADF2_GUI → assets → config.json

In this file, ensure:

- GUI IP: 192.168.1.101

- Port: 7001

These should match the laptop IP and the port on which the GUI is running.

Additionally, another .json configuration file is present in the GUI directory. This file contains multiple IP and port settings. Ensure that:

- GUI IP and port match the laptop running the GUI

  - Example: 192.168.1.101 : 7001

- WebSocket IP and port are consistent with the PC application configuration

  - Example: 192.168.1.101 : 7002

- User access IP is correctly set

  - Example: 192.168.1.101 : 8080

  - This is the IP shown in the terminal and used to open the GUI login page

## 5. Important Guidelines

- Always use the latest released versions of all applications

- Do not randomly modify IP addresses, ports, or flags

- Ensure consistency across PC application, remote server, and GUI configurations

- Incorrect settings may not fail immediately but can cause issues during long-duration operation

If unsure about any configuration, consult the developer before making changes.